- Link Electronics Incorporated

- HDE-3000 Software V1.59

- Upgrade Procedure

You will need to download the UpgradeV159.zip file.  You will need to unzip that file and it will contain three zip files and two documents.  You will need two blank formatted disks and an unzipping utility.  However, you can upgrade without using floppies just unzip the USBv159.zip file into the root directory of a USB flash drive and then perform steps 6 and 8 below.

The following serial numbers: 0103, 0106, 0109, 0115, 0116, 0137, 0139, 0170, 0197, 0198, 0199, or 0200 need too upgrade to at least V1.30 prior upgrading to V1.59.

If you are upgrading a dual unit with versions 1.32 or 1.33 you must adhere to the following page before upgrading you unit.

- Upgrade Procedure:

- Extract files from Disk1v159.zip onto a disk labeled “Disk 1”.

- Extract files from Disk2v159.zip onto a disk labeled “Disk 2”.

- Insert Disk 1 into the disk drive in the HDE-3000.

- Make sure that the HDE-3000 Remote program is running.

- Make the following selection twice “perform software upgrade” in the upgrade software menu on the HDE3000 Remote software.

- When program prompts you to insert disk 2 or when the disk drive LED shuts off, remove Disk 1 and insert Disk 2, then press enter.

- When upgrade complete, remove Disk 2 or USB drive, then shut down and restart the HDE-3000.

- Special V1.59 Upgrading Instructions for HDE3000 Dual Units.

An issue was corrected in those versions to eliminate a timing problem.

To prevent this issue from interfering with the process of upgrading an HDE3000 Dual to V1.59 (from V1.32 or V1.33):

- Disconnect both video inputs.

- Disconnect both LTC time code inputs.

- Don’t upgrade the unit from ENC 2.

If the unit is being fed video or active time code during the software update procedure, the bug may cause the update process to fail.